TLC (Transmit Link Control)

Hierarchy, Functionality, Inputs, Outputs & Configuration

Firmware Rate Scale Manager — WCD WiFi

Technical Reference  •  2026

# Table of Contents

Right-click → Update Field to build the contents.

# Overview

TLC (Rate Scale Manager, “TLC Offload”) is the firmware rate-adaptation engine. It measures link quality and picks the best PHY/MAC transmission parameters — rate, bandwidth, guard interval, spatial streams, aggregation and power — to maximize throughput while keeping the link reliable.

Figure 1. TLC spans four layers; control flows down, statistics flow up.

# 1. Layered Hierarchy

| Layer | Component | Role |
| --- | --- | --- |
| Host / Driver | MmacLinkQuality, DataPathTlcConfig | Sends TLC_MNG_CONFIG_CMD (capabilities); receives TLC_MNG_UPDATE_NTFY |
| Upper MAC | rateScaleMng, rateScaleAggMng | The 'brain': rate / column / BW / GI search + aggregation management |
| Lower MAC | rateScale, tlc | Maps station↔rsId, applies rate to HW retry table, collects TX statistics |
| PHY / HW | rate_n_flags decoder | Turns the encoded rate into a PHY vector (modulation, NSS, BW, GI, antenna) |

Figure 2. Closed-loop adaptation: every statistics report re-tunes the next TX.

# 2. Units & Responsibilities

| Layer / File | Responsibility |
| --- | --- |
| UMAC · rateScaleMng.c | Core algorithm: state machine, column search, up/down-scale, TPC, expected-TPT tables |
| UMAC · rateScaleAggMng.c | Block-ACK / A-MPDU session mgmt (ADDBA/DELBA, window size, agg factor) |
| UMAC · rateScaleMng_static.c | Static tables (expected TPT, columns, XVT fixed-rate path) |
| LMAC · rateScale.c | station↔rsId map, applies rate into HW retry table, agg-allowed decision |
| LMAC · tlc.c | Collects per-station TX statistics, AMSDU BA window, sends TLC_STAT_NTFY |
| PHY | Consumes rate_n_flags; validated against RS_RATE_ERROR_E |
| Driver | TLC_MNG_CONFIG_CMD out, TLC_MNG_UPDATE_NTFY in (legacy MmacLinkQuality) |

## 2.1 Simulation units (USFSTL test framework)

| Sim file | Emulates |
| --- | --- |
| usimLibTlc.c (host) | Host side: send config, set fixed rate, DHC, read notifs, SNR/path-loss sweeps |
| usimLibLmacTlc.c (lmacSim) | LMAC→UMAC: fabricate TX statistics, model path-loss→rate→TPT, capture LQ command |
| lsimLibTlc.c | LMAC LQ config: inject a fixed rate table into LMAC |
| umacSimTlcUTest.c | End-to-end UMAC tests (per-mode configs, fixed rate, TPC, AMSDU, aggregation) |
| umacSimTlcStaticUTest.c | Static-table / XVT-mapping tests |

# 3. TLC Inputs — Origin, Refresh & Usage

Every input, with where the value comes from, how often it refreshes, and what it drives.

| Input | Origin | Refresh rate | Really used — for what |
| --- | --- | --- | --- |
| txed / acked | LMAC tlc.c per BA response | every BA; pushed at ≥20 frames | Raw material for all metrics (not used directly) |
| Success Ratio (SR) | UMAC: 32768×succ/att, EWMA-smoothed | per stats notif (20 / 2000 frames) | PRIMARY metric: up/down-scale, column compare, TPC, AMSDU |
| PER | derived = 1 − SR | with SR | Force-decrease boundary (15% / 90% thresholds) |
| Average TPT | SR × expectedTpt / 32768 | with SR | Compares current vs. candidate column |
| Succ/Fail counters | UMAC totals | each update; reset on search | Trigger a search cycle (4500/400; legacy 480/160) |
| Timestamps | systemTimeGet() | each update | EWMA weighting, stale-data guard, ~10 s idle flush |
| RSSI / SNR | beacon (MAIN_BEACON_DATA_NTFY) | per beacon (~100 ms) | SECONDARY: start-rate pick, TPC gating |
| BA / AMSDU window | TLC_AMSDU_STAT_S (8 BAs) | per BA | AMSDU enable (≥94%) / disable (≤50%), size |
| TB statistics | TLC_TB_STAT_S | per TB (300 ms / 1 s win) | UL-OFDMA (trigger-based) rate path |
| PBO / SAR / thermal | notifications | event-driven (SAR ≥1 s) | Cap usable rate/antenna; drive TPC back-off |

# 4. TLC Outputs — Calculation, Activation & Typical Values

| Output | How calculated | When active | Typical / notes |
| --- | --- | --- | --- |
| Primary rate | Highest-avgTPT rate passing no-decrease SR | always (first HW attempt) | HE/EHT MCS 9–13 in good link; upscale ≤ every 200 ms |
| Retry table | Ladder: primary→MCS-1→MCS-2→SISO/legacy | entry N if all < N failed | ~16 entries; last = robust legacy |
| Bandwidth | column/BW search, clamped to caps/punc | per frame | 20/40/80/160/320 MHz |
| Guard Interval | column attribute (RS_MNG_GI_E) | part of active column | 0.8 µs hi-SR, 3.2 µs robust |
| NSS / column | column-search graph; mimo_delimiter | MIMO if SR+power allow | MIMO strong link; SISO/STBC edge |
| Reduced power (TPC) | 2 dB steps; SR 95/98/10% gates | optimal rate, non-legacy, AMSDU ≥0.5 s | 0–several dB; range −10…+24 dBm |
| Antenna mask | best ant per band | SISO + control frames | A / B / A+B (CDD) |
| RTS protection | rtsBitMsk of protected rates | unless dbg.rtsDisable | on higher/aggregated rates |
| Aggregation | per-mode max; duration 5400 µs | once BA session up | 64–256 frames, ≤5.4 ms |
| A-MSDU | enable ≥94% / disable ≤50% SR | hi-SR aggregating stations | 3.5–11 KB per A-MSDU |
| Rate notif | mirror of primary rate | if driver requested notif | informational to host |

Figure 3. The retry table is a descending rate ladder; lower entries are tried only on failure.

# 5. Configuration & Tunable Knobs

## 5.1 Station configuration — TLC_MNG_CONFIG_CMD (from driver)

| Field | Controls |
| --- | --- |
| maxChWidth | Max bandwidth (20…320 MHz) |
| bestSuppMode | Highest PHY mode (Legacy/HT/VHT/HE/EHT/UHR) |
| chainsEnabled | Antenna chains (A/B) → SISO vs. MIMO |
| sgiChWidthSupport | SGI allowed per bandwidth (HT/VHT) |
| configFlags | STBC, LDPC, HE-DCM, EHT-DUP, 2×LTF block, ELR |
| nonHt | Bitmap of allowed CCK/OFDM legacy rates |
| mcs[nss][bw] | Per-NSS / per-BW supported MCS bitmap |
| maxMpduLen | Drives max A-MSDU size (0 = AMSDU off) |
| maxTxOp | TXOP cap for all ACs (0 = no limit) |

## 5.2 Runtime debug hooks — TLC_MNG_DEBUG_CMD / DHC

| Type | Effect |
| --- | --- |
| FIXED_RATE | Force a fixed rate_n_flags (0 = auto) |
| PARTIAL_FIXED_RATE | Fix some fields, scale the rest |
| AGG_DURATION_LIM | A-MPDU duration (100–8000 µs) |
| AGG_FRAME_CNT_LIM | A-MPDU frame count (1–64) |
| TPC_ENABLED | Enable/disable TPC (default on) |
| TPC_STATS | Return per-TPC-step histogram |
| RTS_DISABLE | Disable RTS protection |
| FAST_START | Start at max configured PHY rate (WFA) |

Internal-only (≥128): MAX_BW, AGG_CONTROL, AGG_F_THRESH, 2ANT_LIMIT, FAST_START, FIXED_AMSDU, BT_SAR_DICTATE_MODE. Global debug state g_rsMng.dbg holds fastStart, rtsDisable, amsdu.size/tids, maxBw, durationLimit, sarValue.

## 5.3 TPC configuration — TPC_CONFIG_S

| Field | Default | Meaning |
| --- | --- | --- |
| tpcEnabled | Auto | 0 Auto / 1 Force-off / 2 Force-on |
| tpcSrIncrease | 98% | SR to step up power back-off |
| tpcSrDecrease | 95% | SR to step down power back-off |
| tpcSrForceDisable | 10% | SR to jump back to full power |
| tpcPwrBoStepSize | 2 dB | Back-off step |
| tpcSearchCycleExtraBo | 2 dB | Extra back-off during search |

## 5.4 Compile-time algorithm constants (_rateScaleMng.h)

| Constant | Value | Role |
| --- | --- | --- |
| RS_MNG_PERFECT_SR | 95% | 'perfect' link threshold |
| RS_MNG_SR_NO_DECREASE | 90% | above this, never downscale |
| RS_MNG_SR_FORCE_DECREASE | 15% | below this, force downscale |
| RS_STAT_THOLD | 20 | frames per stats notif (normal) |
| RS_MNG_UPSCALE_AGG_FRAME_COUNT | 20 | frames in a test window |
| RS_MNG_SHORT_AGG_FRAME_COUNT | 10 | frames in fast-start window |
| RS_MNG_OPTIMAL_RATE_FRAME_COUNT | 2000 | frames between checks at optimal rate |
| RS_MNG_UPSCALE_MAX_FREQUENCY | 200 ms | min time between upscale attempts |
| RS_MNG_UPSCALE_SEARCH_CYCLE_MAX_FREQ | 300 ms | min time between search cycles |
| RS_MNG_AMSDU_SR_ENABLE / DISABLE | 94% / 50% | AMSDU gating |
| RS_MNG_AGG_DURATION_LIMIT | 5400 µs | default A-MPDU duration |
| RS_MNG_EWMA_DENOMINATOR | 256 | SR smoothing resolution |

# 6. Rate-Scale State Machine

Figure 4. Three states: steady rate, column search, and power (TPC) search.

| State | Meaning | Exit |
| --- | --- | --- |
| STAY_IN_COLUMN | Steady rate; STAY / UPSCALE / DOWNSCALE | → SEARCH on succ/fail/time |
| SEARCH_CYCLE | Probe columns/BW; compare averageTpt | → STAY or → TPC_SEARCH |
| TPC_SEARCH | Reduce power at the optimal rate | → STAY when done |

# 7. Reference Data Tables (Appendix)

## 7.1 MCS — Modulation & Coding

| MCS | Modulation | Coding | MCS | Modulation | Coding |
| --- | --- | --- | --- | --- | --- |
| 0 | BPSK | 1/2 | 8 | 256-QAM | 3/4 |
| 1 | QPSK | 1/2 | 9 | 256-QAM | 5/6 |
| 2 | QPSK | 3/4 | 10 | 1024-QAM | 3/4 |
| 3 | 16-QAM | 1/2 | 11 | 1024-QAM | 5/6 |
| 4 | 16-QAM | 3/4 | 12 | 4096-QAM | 3/4 |
| 5 | 64-QAM | 2/3 | 13 | 4096-QAM | 5/6 |
| 6 | 64-QAM | 3/4 | 14 | DUP DCM | — |
| 7 | 64-QAM | 5/6 | 15 | DCM | — |

## 7.2 Expected Throughput — Legacy (CCK / OFDM), effective Mbps

| Rate | Mbps | Rate | Mbps | Rate | Mbps |
| --- | --- | --- | --- | --- | --- |
| CCK 1M | 1 | OFDM 6M | 4 | OFDM 24M | 14 |
| CCK 2M | 1 | OFDM 9M | 6 | OFDM 36M | 18 |
| CCK 5.5M | 3 | OFDM 12M | 8 | OFDM 48M | 21 |
| CCK 11M | 5 | OFDM 18M | 11 | OFDM 54M | 22 |

Values are effective throughput (raw table stores Mbps×10). Source: expectedTptNonHt.

## 7.3 Expected Throughput — HT/VHT, 80 MHz (effective Mbps)

| Config | M0 | M1 | M2 | M3 | M4 | M5 | M6 | M7 | M8 | M9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SISO NGI | 24 | 48 | 72 | 96 | 143 | 185 | 204 | 223 | 259 | 281 |
| SISO SGI | 26 | 53 | 80 | 106 | 158 | 202 | 223 | 243 | 281 | 305 |
| MIMO NGI | 48 | 95 | 142 | 184 | 258 | 323 | 352 | 380 | 430 | 460 |
| MIMO SGI | 53 | 106 | 157 | 202 | 281 | 349 | 380 | 409 | 461 | 492 |

MCS 0–9, 80 MHz. Source: expectedTptHtVht[CHANNEL_WIDTH80].

## 7.4 Expected Throughput — HE, 80 MHz (effective Mbps)

| Config | M0 | M1 | M2 | M3 | M4 | M5 | M6 | M7 | M8 | M9 | M10 | M11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SISO 3.2µs | 26 | 52 | 78 | 104 | 156 | 208 | 234 | 260 | 312 | 347 | 390 | 434 |
| SISO 1.6µs | 29 | 58 | 87 | 116 | 174 | 231 | 260 | 289 | 347 | 386 | 434 | 482 |
| SISO 0.8µs | 31 | 61 | 92 | 122 | 184 | 245 | 276 | 306 | 368 | 408 | 459 | 510 |
| MIMO 3.2µs | 52 | 104 | 156 | 208 | 312 | 416 | 468 | 521 | 625 | 694 | 781 | 868 |
| MIMO 1.6µs | 58 | 116 | 174 | 231 | 347 | 463 | 521 | 578 | 694 | 771 | 868 | 964 |
| MIMO 0.8µs | 61 | 122 | 184 | 245 | 368 | 490 | 551 | 612 | 735 | 817 | 919 | 1021 |

MCS 0–11, 80 MHz. Source: expectedTptHe[CHANNEL_WIDTH80].

Figure 5. HE 80 MHz, GI 0.8 µs — MIMO roughly doubles SISO throughput.

## 7.5 Rate-Scale Columns (RS_MNG_COLUMN_DESC_E)

| Group | Columns |
| --- | --- |
| Legacy (CCK/OFDM) | NON_HT_ANT_A, NON_HT_ANT_B |
| HT/VHT SISO | SISO_STBC, SISO_ANT_A, SISO_ANT_B (+ _SGI variants) |
| HT/VHT MIMO | MIMO, MIMO_SGI |
| HE SISO | HE_{3.2/1.6/0.8}_SISO_{STBC/ANT_A/ANT_B} |
| HE MIMO | HE_{3.2/1.6/0.8}_MIMO |

A column = a fixed combination of mode, NSS, GI and antenna. The search cycle walks a graph of related columns (nextCols) and compares averageTpt to find the best one.

# 8. Quick File Map

| Layer | Path |
| --- | --- |
| UMAC algorithm | fw/src/umac/main/dataPath/rateScaleMng/ |
| LMAC apply | fw/src/lmac/mcm/core/dataPath/rateScale/ |
| LMAC statistics | fw/src/lmac/mcm/core/dataPath/tlc/ |
| API config/notif | fw/inc/commands/apiGroupDatapath.h |
| API LQ/retry table | fw/src/shared/inc/apiLmacUmac.h, stationDefines.h |
| Host sim | softwareTesting/.../umac/libs/host/usimLibTlc.c |
| LMAC sim | softwareTesting/.../umac/libs/lmacSim/usimLibLmacTlc.c |
| UMAC tests | softwareTesting/projects/usim/umacSimTlc/umacSimTlcUTest.c |
| Driver config | wifi_drv-dev/.../DataPath/DataPathTlcConfig/DataPathTlcConfig.c |
| Driver legacy TLC | wifi_drv-dev/.../MMAC/MmacLinkQuality/ |